Dam safety buffer and settlement exposure

Dam failure is among the most consequential infrastructure hazards, capable of releasing sudden flood volumes that overwhelm downstream settlements within minutes. Across Ghana, 18 registered dams sit within a national grid of over 25,000 settlements, yet the spatial relationship between dam locations and proximate populations remains unquantified. You are required to produce a dam safety buffer and settlement exposure map that delineates concentric inundation risk zones around every dam and identifies which settlements fall within each zone, delivering a spatially accurate, decision-ready dataset that disaster risk managers, infrastructure planners, and emergency response coordinators can use to prioritise preparedness and evacuation planning.

You will work with three layers: dams.gpkg (point layer of 18 dams across Ghana with attributes including NAME, HEIGHT, CAPACITY, and RIVER), settlements.gpkg (point layer of 25,342 settlements with population and service attributes), and rivers.gpkg (line layer of 63 river segments). All layers are in EPSG:25000 (Leigon / Ghana Metre Grid), a metric projected CRS suited for distance-based analysis. No reprojection is required.

The inundation geometry is built by generating three concentric rings around every dam point using a Multi-Ring Constant Buffer with a step distance of 5,000 metres and a ring count of 3. The result is 54 buffer polygons (18 dams x 3 rings), each carrying a ringId field (1, 2, or 3) and a distance field (5000, 10000, 15000), representing Zone_1 (0-5 km), Zone_2 (5-10 km), and Zone_3 (10-15 km). This output is saved as dam_inundation_buffers.gpkg, and zone thresholds must not be altered.

The primary deliverable is settlements_exposure_tagged.gpkg: a point layer retaining all 25,342 features from settlements.gpkg with two additional text fields. inundation_zone holds the assigned zone label string, or Outside inundation zone for settlements beyond all rings, assigned by evaluating each settlement against all 54 buffer polygons and prioritising the minimum ringId where multiple intersections occur. nearest_dam holds the NAME value of the dam from the intersecting buffer. Feature count must remain exactly 25,342. A filtered subset, settlements_within_inundation.gpkg, is produced from the tagged layer containing only features where inundation_zone is not Outside inundation zone and must contain exactly 1,790 features.

In the project, settlements_exposure_tagged.gpkg is classified on inundation_zone using red circles for Zone_1, orange circles for Zone_2, blue circles for Zone_3, and gray circles for Outside inundation zone. dam_inundation_buffers.gpkg is symbolised by ringId with semi-transparent fills and matching coloured outlines. dams.gpkg is rendered as red star markers and rivers.gpkg as blue lines.

The map output is an A4 landscape layout named Dam_Safety_Buffer_Settlement_Exposure, exported as Dam_Safety_Buffer_Settlement_Exposure.pdf at 300 DPI, with settlements_exposure_tagged.gpkg as the primary thematic layer and dam_inundation_buffers.gpkg as the zone reference. The layout carries the title Ghana Dam Safety Buffers and Settlement Exposure, a legend covering all four inundation zone labels with colour swatches and the dam star symbol, a north arrow, a scale bar in kilometres, a CRS note reading EPSG:25000 - Leigon / Ghana Metre Grid, and a data source note naming dams.gpkg, settlements.gpkg, and rivers.gpkg.

Deliver a project folder named Dam safety buffer and settlement exposure map containing the three input layers (dams.gpkg, settlements.gpkg, rivers.gpkg), three derived outputs (dam_inundation_buffers.gpkg, settlements_exposure_tagged.gpkg, settlements_within_inundation.gpkg), Dam_Safety_Buffer_Settlement_Exposure.pdf, and the project file Dam_Safety_Buffer_Settlement_Exposure.qgz saved with relative paths. The project must reopen without broken layer links, CRS mismatches, or missing files. The reference_layout.png only provides a visual guide and is not to be loaded in the project or used in any of the analyses.